### What We Built

A smart safety helmet for construction workers that:

- Detects falls using a 3-phase algorithm (no false alarms)

- Tracks location via GPS with indoor fallback

- Analyzes fatigue using an AI engine

- Maps dangerous zones on the worksite

- Saves data offline when WiFi cuts out

- Shows everything on a live web dashboard

### Hardware Used

| Component | Job | Pin |
| --- | --- | --- |
| ESP32 | Main brain + WiFi | — |
| MPU-6050 | Motion sensor | SDA=21, SCL=22 |
| NEO-6M GPS | Location | RX2=16, TX2=17 |
| Active Buzzer | Alarm sound | GPIO 25 |
| 18650 Battery | Power | — |
| TP4056 | Charging | — |

### Libraries Needed (install in Arduino Library Manager)

✅ Adafruit MPU6050

✅ Adafruit Unified Sensor

✅ TinyGPSPlus

✅ Preferences       ← built into ESP32, no install needed

#include <Adafruit_MPU6050.h>

#include <Adafruit_Sensor.h>

#include <TinyGPSPlus.h>

#include <WiFi.h>

#include <WebServer.h>

#include <Wire.h>

#include <HardwareSerial.h>

#include <Preferences.h>

#include <math.h>

Adafruit_MPU6050 mpu;

TinyGPSPlus      gps;

HardwareSerial   gpsSerial(2);

WebServer        server(80);

Preferences      prefs;

const char* SSID     = "HelmetSystem";

const char* PASSWORD = "12345678";

const int BUZZER_PIN = 25;

const int BUTTON_PIN = 4;

const int SWITCH_PIN = 13;

const char* WORKER_ID   = "W-01";

const char* WORKER_NAME = "Worker 1";

const float    FF_THR     =  4.9f;

const float    IMP_THR    = 24.5f;

const float    STILL_DEV  =  2.94f;

const uint32_t FF_MIN_MS  =  80;

const uint32_t IMP_MAX_MS = 500;

const uint32_t STILL_MS   = 600;

const int   AI_BUF_SIZE  = 50;

const float FAT_ACT_THR  =  1.18f;

const float FAT_TILT_THR = 25.0f;

const int   FAT_DUR_S    = 30;

enum FallPhase { FP_IDLE, FP_FREEFALL, FP_IMPACT, FP_CONFIRM };

FallPhase fallPhase    = FP_IDLE;

uint32_t  ffStartMs    = 0;

uint32_t  impStartMs   = 0;

uint32_t  stillStartMs = 0;

float ax = 0, ay = 0, az = 0;

float gx = 0, gy = 0, gz = 0;

float totalAcc = 9.81f;

float tiltDeg  = 0.0f;

float    lastValidLat = 0.0f;

float    lastValidLng = 0.0f;

bool     gpsValid     = false;

uint32_t gpsLostMs    = 0;

bool fallDetected = false;

bool sosActive    = false;

bool fatigueAlert = false;

int  fatigueScore = 0;

bool helmetWorn   = false;

bool systemActive = false;

int fallCount = 0;

int sosCount  = 0;

String alertLog = "";

float aiBuf[AI_BUF_SIZE];

int   aiBufIdx   = 0;

int   aiBufCount = 0;

struct Zone { float lat, lng; int cnt; char name[12]; };

Zone zones[5];

int  zoneCount = 0;

bool     buzOn      = false;

uint32_t buzStart   = 0;

int      buzPattern = 0;

uint32_t lastAiMs       = 0;

uint32_t lastFatAlert   = 0;

int      continuousFatS = 0;

int      offlineQueue   = 0;

uint32_t bootTime       = 0;

uint32_t switchOnTime   = 0;

void addLog(const String& msg) {

alertLog = msg + "||" + alertLog;

int sep = 0, pos = 0;

while ((pos = alertLog.indexOf("||", pos)) != -1 && sep < 9) {

sep++; pos += 2;

}

if (sep >= 9 && pos != -1) alertLog = alertLog.substring(0, pos);

}

void startupBeep() {

digitalWrite(BUZZER_PIN, HIGH); delay(100);

digitalWrite(BUZZER_PIN, LOW);  delay(100);

digitalWrite(BUZZER_PIN, HIGH); delay(100);

digitalWrite(BUZZER_PIN, LOW);  delay(100);

digitalWrite(BUZZER_PIN, HIGH); delay(300);

digitalWrite(BUZZER_PIN, LOW);

}

void shutdownBeep() {

digitalWrite(BUZZER_PIN, HIGH); delay(500);

digitalWrite(BUZZER_PIN, LOW);  delay(100);

digitalWrite(BUZZER_PIN, HIGH); delay(200);

digitalWrite(BUZZER_PIN, LOW);

}

void saveSessionData() {

prefs.begin("session", false);

prefs.putInt("fall_cnt", fallCount);

prefs.putInt("sos_cnt",  sosCount);

prefs.putInt("zone_cnt", zoneCount);

for (int i = 0; i < zoneCount; i++) {

String key = "z" + String(i);

String val = String(zones[i].lat, 6) + "," +

String(zones[i].lng, 6) + "," +

String(zones[i].cnt);

prefs.putString(key.c_str(), val);

}

prefs.end();

}

void loadSessionData() {

prefs.begin("session", true);

fallCount = prefs.getInt("fall_cnt", 0);

sosCount  = prefs.getInt("sos_cnt",  0);

zoneCount = prefs.getInt("zone_cnt", 0);

for (int i = 0; i < zoneCount && i < 5; i++) {

String key = "z" + String(i);

String val = prefs.getString(key.c_str(), "");

if (val.length() > 0) {

int c1 = val.indexOf(',');

int c2 = val.indexOf(',', c1 + 1);

zones[i].lat = val.substring(0, c1).toFloat();

zones[i].lng = val.substring(c1 + 1, c2).toFloat();

zones[i].cnt = val.substring(c2 + 1).toInt();

snprintf(zones[i].name, 12, "Zone %c", 'A' + i);

}

}

prefs.end();

Serial.printf("[BOOT] Loaded: %d falls %d SOS %d zones\n",

fallCount, sosCount, zoneCount);

}

void clearSessionData() {

prefs.begin("session", false);

prefs.clear();

prefs.end();

fallCount = 0;

sosCount  = 0;

zoneCount = 0;

alertLog  = "";

Serial.println("[SESSION] Cleared");

}

void saveOffline(const char* type, float lat, float lng) {

prefs.begin("helm", false);

int n = prefs.getInt("n", 0);

if (n < 20) {

String v = String(type) + "," + String(lat, 6) + "," + String(lng, 6);

prefs.putString(("e" + String(n)).c_str(), v);

prefs.putInt("n", n + 1);

offlineQueue = n + 1;

}

prefs.end();

}

void flushOfflineEvents() {

prefs.begin("helm", false);

int n = prefs.getInt("n", 0);

if (n > 0) {

Serial.printf("[WiFi] Flushing %d stored events\n", n);

prefs.clear();

offlineQueue = 0;

}

prefs.end();

}

void updateZone(float lat, float lng) {

if (lat == 0 && lng == 0) return;

const float R = 0.0002f;

for (int i = 0; i < zoneCount; i++) {

float d = sqrtf(powf(lat - zones[i].lat, 2) + powf(lng - zones[i].lng, 2));

if (d < R) { zones[i].cnt++; saveSessionData(); return; }

}

if (zoneCount < 5) {

zones[zoneCount].lat = lat;

zones[zoneCount].lng = lng;

zones[zoneCount].cnt = 1;

snprintf(zones[zoneCount].name, 12, "Zone %c", 'A' + zoneCount);

zoneCount++;

saveSessionData();

}

}

String zonesJSON() {

String j = "[";

for (int i = 0; i < zoneCount; i++) {

if (i) j += ",";

j += "{\"n\":\""; j += zones[i].name;

j += "\",\"lat\":";  j += String(zones[i].lat, 6);

j += ",\"lng\":";    j += String(zones[i].lng, 6);

j += ",\"c\":";      j += zones[i].cnt; j += "}";

}

return j + "]";

}

void readGPS() {

while (gpsSerial.available()) gps.encode(gpsSerial.read());

if (gps.location.isValid() && gps.location.age() < 2000) {

lastValidLat = gps.location.lat();

lastValidLng = gps.location.lng();

gpsValid     = true;

gpsLostMs    = 0;

} else {

gpsValid = false;

if (!gpsLostMs) gpsLostMs = millis();

}

}

void detectHelmetWorn() {

helmetWorn = (fabsf(totalAcc - 9.81f) < 3.0f && tiltDeg < 60.0f);

}

void detectFall() {

uint32_t now = millis();

switch (fallPhase) {

case FP_IDLE:

if (totalAcc < FF_THR) {

fallPhase = FP_FREEFALL;

ffStartMs = now;

}

break;

case FP_FREEFALL:

if (totalAcc >= FF_THR) {

if (now - ffStartMs >= FF_MIN_MS) {

fallPhase  = FP_IMPACT;

impStartMs = now;

} else {

fallPhase = FP_IDLE;

}

}

if (now - ffStartMs > 1500) fallPhase = FP_IDLE;

break;

case FP_IMPACT:

if (totalAcc > IMP_THR) {

stillStartMs = now;

fallPhase    = FP_CONFIRM;

}

if (now - impStartMs > IMP_MAX_MS) fallPhase = FP_IDLE;

break;

case FP_CONFIRM:

if (fabsf(totalAcc - 9.81f) < STILL_DEV) {

if (now - stillStartMs >= STILL_MS && !fallDetected) {

fallDetected = true;

fallCount++;

digitalWrite(BUZZER_PIN, HIGH);

buzOn = true; buzStart = now; buzPattern = 0;

String entry = "FALL #" + String(fallCount) +

" | " + String(lastValidLat, 5) + "," + String(lastValidLng, 5);

addLog(entry);

saveOffline("FALL", lastValidLat, lastValidLng);

updateZone(lastValidLat, lastValidLng);

saveSessionData();

Serial.println("[FALL] CONFIRMED");

}

} else {

if (now - stillStartMs < 200) fallPhase = FP_IDLE;

}

break;

}

}

void runAI() {

if (aiBufCount < AI_BUF_SIZE) return;

if (millis() - lastAiMs < 2500) return;

lastAiMs = millis();

float sum = 0;

for (int i = 0; i < AI_BUF_SIZE; i++) sum += fabsf(aiBuf[i] - 9.81f);

float avgAct = sum / AI_BUF_SIZE;

float var = 0;

for (int i = 0; i < AI_BUF_SIZE; i++) {

float d = fabsf(aiBuf[i] - 9.81f) - avgAct;

var += d * d;

}

var /= AI_BUF_SIZE;

float tilt  = tiltDeg;

float score = 0;

if (avgAct < FAT_ACT_THR)  score += (1.0f - avgAct / FAT_ACT_THR) * 40.0f;

if (tilt   > FAT_TILT_THR) score += constrain((tilt - FAT_TILT_THR) / 20.0f * 35.0f, 0.0f, 35.0f);

if (var    < 0.05f)        score += 25.0f;

fatigueScore = (int)constrain(score, 0, 100);

if (fatigueScore > 60) {

continuousFatS += 2;

if (continuousFatS >= FAT_DUR_S && millis() - lastFatAlert > 120000UL) {

fatigueAlert = true;

lastFatAlert = millis();

buzOn = true; buzStart = millis(); buzPattern = 2;

addLog("FATIGUE | act:" + String(avgAct, 2) +

" tilt:" + String(tilt, 0) + "deg sc:" + String(fatigueScore));

}

} else {

continuousFatS = max(0, continuousFatS - 1);

if (fatigueScore < 30) fatigueAlert = false;

}

Serial.printf("[AI] act=%.2f tilt=%.0f var=%.4f score=%d worn=%s\n",

avgAct, tilt, var, fatigueScore, helmetWorn ? "YES" : "NO");

}

void controlBuzzer() {

if (!buzOn) { digitalWrite(BUZZER_PIN, LOW); return; }

uint32_t e = millis() - buzStart;

int c;

switch (buzPattern) {

case 0:

if (e < 10000) { c = e % 300; digitalWrite(BUZZER_PIN, c < 200 ? HIGH : LOW); }

else buzOn = false;

break;

case 1:

if (e < 15000) { c = e % 1000; digitalWrite(BUZZER_PIN, c < 600 ? HIGH : LOW); }

else buzOn = false;

break;

case 2:

if (e < 5000) { c = e % 1000; digitalWrite(BUZZER_PIN, c < 500 ? HIGH : LOW); }

else buzOn = false;

break;

}

}

void systemStart() {

systemActive  = true;

switchOnTime  = millis();

fallDetected  = false;

sosActive     = false;

fatigueAlert  = false;

fallPhase     = FP_IDLE;

fatigueScore  = 0;

aiBufIdx      = 0;

aiBufCount    = 0;

buzOn         = false;

digitalWrite(BUZZER_PIN, LOW);

loadSessionData();

startupBeep();

addLog("SYSTEM ON");

Serial.println("[SWITCH] Monitoring started");

}

void systemStop() {

systemActive = false;

fallPhase    = FP_IDLE;

buzOn        = false;

digitalWrite(BUZZER_PIN, LOW);

saveSessionData();

shutdownBeep();

addLog("SYSTEM OFF");

Serial.println("[SWITCH] Monitoring stopped - data saved");

}

void handleData() {

server.sendHeader("Access-Control-Allow-Origin", "*");

server.sendHeader("Cache-Control", "no-cache");

uint32_t lostS     = gpsLostMs ? (millis() - gpsLostMs) / 1000 : 0;

uint32_t uptimeSec = systemActive ? (millis() - switchOnTime) / 1000 : 0;

String j = "{";

j += "\"worker_id\":\"";    j += WORKER_ID;   j += "\"";

j += ",\"worker_name\":\""; j += WORKER_NAME; j += "\"";

j += ",\"system_active\":"; j += systemActive ? "true" : "false";

j += ",\"ax\":";  j += String(ax, 2);

j += ",\"ay\":";  j += String(ay, 2);

j += ",\"az\":";  j += String(az, 2);

j += ",\"mag\":"; j += String(totalAcc, 2);

j += ",\"tilt\":"; j += String(tiltDeg, 1);

j += ",\"gx\":";  j += String(gx, 1);

j += ",\"gy\":";  j += String(gy, 1);

j += ",\"gz\":";  j += String(gz, 1);

j += ",\"lat\":"; j += String(lastValidLat, 6);

j += ",\"lng\":"; j += String(lastValidLng, 6);

j += ",\"gps_valid\":";   j += gpsValid ? "true" : "false";

j += ",\"gps_cached\":";  j += (!gpsValid && lastValidLat != 0) ? "true" : "false";

j += ",\"gps_lost_s\":";  j += lostS;

j += ",\"fall\":";        j += fallDetected ? "true" : "false";

j += ",\"fall_phase\":";  j += (int)fallPhase;

j += ",\"sos\":";         j += sosActive    ? "true" : "false";

j += ",\"fatigue\":";     j += fatigueAlert ? "true" : "false";

j += ",\"fat_score\":";   j += fatigueScore;

j += ",\"fall_cnt\":";    j += fallCount;

j += ",\"sos_cnt\":";     j += sosCount;

j += ",\"offline_q\":";   j += offlineQueue;

j += ",\"helmet_worn\":"; j += helmetWorn ? "true" : "false";

j += ",\"uptime_s\":";    j += uptimeSec;

j += ",\"log\":\"";       j += alertLog; j += "\"";

j += ",\"zones\":";       j += zonesJSON();

j += "}";

server.send(200, "application/json", j);

}

void handleRoot() {

uint32_t uptimeSec = systemActive ? (millis() - switchOnTime) / 1000 : 0;

int h = uptimeSec / 3600;

int m = (uptimeSec % 3600) / 60;

int s = uptimeSec % 60;

String html = "<!DOCTYPE html><html><head><meta charset='UTF-8'>";

html += "<meta name='viewport' content='width=device-width,initial-scale=1'>";

html += "<meta http-equiv='refresh' content='5'>";

html += "<title>Smart Helmet</title><style>";

html += "body{font-family:sans-serif;background:#0a0e1a;color:#dce8f5;text-align:center;padding:30px;}";

html += "h1{color:#ffab00;}.safe{color:#00e5b0;}.danger{color:#ff3d6b;}.warn{color:#ffab00;}";

html += ".btn{display:inline-block;margin:8px;padding:12px 22px;border-radius:8px;";

html += "background:#1a2840;color:#dce8f5;text-decoration:none;border:1px solid #1e2d47;}";

html += ".btn.red{border-color:rgba(255,61,107,.6);color:#ff3d6b;}";

html += ".btn.orange{border-color:rgba(255,171,0,.6);color:#ffab00;}";

html += "p{font-size:1.1rem;margin:6px 0;}</style></head><body>";

html += "<h1>" + String(WORKER_NAME) + " - " + String(WORKER_ID) + "</h1>";

if (!systemActive) {

html += "<p class='warn'>SYSTEM OFF - flip switch to start monitoring</p>";

} else {

html += "<p class='safe'>SYSTEM ON - monitoring active</p>";

html += helmetWorn ? "<p class='safe'>Helmet: ON HEAD</p>" : "<p class='warn'>Helmet: NOT WORN</p>";

html += fallDetected ? "<p class='danger'>FALL DETECTED!</p>" : "<p class='safe'>Worker Safe</p>";

html += "<p>Fatigue Score: <b>" + String(fatigueScore) + "%</b></p>";

html += "<p>Falls this session: <b>" + String(fallCount) + "</b></p>";

html += "<p>Session time: <b>" + String(h) + "h " + String(m) + "m " + String(s) + "s</b></p>";

if (gpsValid)

html += "<p>GPS: " + String(lastValidLat, 5) + ", " + String(lastValidLng, 5) + "</p>";

else if (lastValidLat != 0)

html += "<p>GPS cached: " + String(lastValidLat, 5) + ", " + String(lastValidLng, 5) + "</p>";

else

html += "<p>Searching for GPS...</p>";

}

html += "<br><a class='btn red' href='/reset'>Reset Alert</a>";

html += "<a class='btn orange' href='/clearsession'>Clear Session</a>";

html += "<a class='btn' href='/data'>Raw JSON</a>";

html += "</body></html>";

server.send(200, "text/html", html);

}

void handleReset() {

fallDetected = false;

sosActive    = false;

fatigueAlert = false;

fallPhase    = FP_IDLE;

buzOn        = false;

digitalWrite(BUZZER_PIN, LOW);

server.sendHeader("Location", "/");

server.send(303);

}

void handleClearSession() {

clearSessionData();

server.sendHeader("Location", "/");

server.send(303);

}

void setup() {

Serial.begin(115200);

delay(2000);

bootTime = millis();

Serial.println("[BOOT] Smart Helmet ready");

pinMode(BUZZER_PIN, OUTPUT);

pinMode(BUTTON_PIN, INPUT_PULLUP);

pinMode(SWITCH_PIN, INPUT_PULLUP);

digitalWrite(BUZZER_PIN, LOW);

Wire.begin();

if (!mpu.begin()) {

Serial.println("[ERROR] MPU6050 not found!");

while (1) {

digitalWrite(BUZZER_PIN, HIGH); delay(200);

digitalWrite(BUZZER_PIN, LOW);  delay(200);

}

}

mpu.setFilterBandwidth(MPU6050_BAND_44_HZ);

Serial.println("[OK] MPU6050 ready");

gpsSerial.begin(9600, SERIAL_8N1, 16, 17);

Serial.println("[OK] GPS ready");

WiFi.softAP(SSID, PASSWORD);

Serial.print("[OK] AP IP: ");

Serial.println(WiFi.softAPIP());

server.on("/",             handleRoot);

server.on("/reset",        handleReset);

server.on("/data",         handleData);

server.on("/clearsession", handleClearSession);

server.begin();

Serial.println("[OK] Server ready - waiting for switch");

if (digitalRead(SWITCH_PIN) == LOW) {

systemStart();

}

}

void loop() {

server.handleClient();

static bool lastSwitch = HIGH;

bool curSwitch = digitalRead(SWITCH_PIN);

if (lastSwitch == HIGH && curSwitch == LOW) {

systemStart();

}

if (lastSwitch == LOW && curSwitch == HIGH) {

systemStop();

}

lastSwitch = curSwitch;

if (!systemActive) {

delay(100);

return;

}

sensors_event_t accel, gyro, temp;

mpu.getEvent(&accel, &gyro, &temp);

ax = accel.acceleration.x;

ay = accel.acceleration.y;

az = accel.acceleration.z;

gx = gyro.gyro.x * RAD_TO_DEG;

gy = gyro.gyro.y * RAD_TO_DEG;

gz = gyro.gyro.z * RAD_TO_DEG;

totalAcc = sqrtf(ax*ax + ay*ay + az*az);

tiltDeg  = degrees(acosf(constrain(az / totalAcc, -1.0f, 1.0f)));

aiBuf[aiBufIdx] = totalAcc;

aiBufIdx = (aiBufIdx + 1) % AI_BUF_SIZE;

if (aiBufCount < AI_BUF_SIZE) aiBufCount++;

detectHelmetWorn();

detectFall();

runAI();

readGPS();

controlBuzzer();

static bool lastBtn = HIGH;

bool curBtn = digitalRead(BUTTON_PIN);

if (lastBtn == HIGH && curBtn == LOW) {

sosActive = true;

sosCount++;

buzOn = true; buzStart = millis(); buzPattern = 1;

addLog("SOS #" + String(sosCount));

saveSessionData();

Serial.println("[SOS] Manual SOS pressed!");

}

lastBtn = curBtn;

static uint32_t lastDbg = 0;

if (millis() - lastDbg > 1000) {

Serial.printf("acc=%.2f tilt=%.0f gps=%s fat=%d%% phase=%d fall=%s worn=%s\n",

totalAcc, tiltDeg,

gpsValid ? "LIVE" : (lastValidLat ? "CACHED" : "SEARCH"),

fatigueScore, (int)fallPhase,

fallDetected ? "YES" : "no",

helmetWorn ? "YES" : "no");

lastDbg = millis();

}

delay(20);

}